Hi Emannuel,

Keith and I tried coming up with a way on how a bad setting on and 
original VSFs could be causing a steady 278 output for all three 
channels, but we couldn't.  Hopefully resetting these flash values will 
recover the meters operation.

I'm attaching the most current version of ECO Host, used for the VSF 
meters.  I have not had any success in getting it to run so I don't know 
if it will be of use to you or not.

For using a terminal program, here is what I've found.

1.  Right after power up (within 1 second), press the 'p' key.  This 
will stop the data processing and send the flash settings.
2.  Most of the settings are set by pressing the 'b' key.  You will get 
the prompt:

SGALWM.

Just type the 100010 (one letter for each setting) and press Enter.

4.  The other 2 settings are;

D    0, and T   30.

To set these two parameters, type the letter 'd' or 't' and enter 0 or 
30 at the prompt.

If you have any further question about what I've sent, email me at 
romanko@teleport.com.  I'll be checking my home email off and on.

Good Luck,